CHAPTER 1

LOAD FREQUENCY CONTROL IN A SINGLE AREA POWER SYSTEM.

##### INTRODUCTION

Power system is used for the conversion of natural energy to electric energy. For the optimization of electrical equipment, it is necessary to ensure the electric power quality. It is known three phase AC is used for transportation of electricity. During the transportation, both the active and reactive power balance must be maintained between the generation and utilization AC power. When either frequency or voltage changes equilibrium point will shift. Good quality of electrical power system means both the voltage and frequency to be fixed at desired values irrespective of change in loads that occurs randomly. It is in fact impossible to maintain both active and reactive power without control which would result in variation of voltage and frequency levels. To cancel the effect of load variation and to keep frequency and voltage level constant a control system is required. Though the active and reactive powers have a combined effect on the frequency and voltage, the control problem of the frequency and voltage can be separated. Frequency is mostly dependent on the active power and voltage is mostly dependent on the reactive power. Thus, the issue of controlling power systems can be separated into two independent problems. The active power and frequency control are called as load frequency control (LFC). The most important task of LFC is to maintain the frequency constant against the varying active power loads, which is also referred as un- known external disturbance.

Power exchange error is an important task of LFC. Generally, a power system is composed of several generating units. To improve the fault tolerance of the whole power system, these generating units are connected through tie-lines. This use of tie-line power creates a new error in the control problem, which is the tie-line power exchange error. When sudden change in active power load occurs to an area, the area will get its energy through tie-lines from other areas. Eventually the area that is subject to the change in load should balance it without external support. Or else there will be economic conflicts between the areas. This is why each area requires separate load frequency controller to regulate the tie line power exchange error so that all the areas in an interconnected system can set their set points differently. In short, the LFC has two major duties, which are to maintain the desired value of frequency and also to keep the tie line power exchange under schedule in the presence of any load changes. Also, the LFC has to be unaffected by unknown external disturbances and system model and parameter variation.

##### 1.1.1 REASONS FOR THE NEED OF MAINTAINING CONSTANTFREQUENCY:

- The speed of ac. motors are directly related to the frequency.

- If the normal operating frequency is 50 Hz and the turbines run at speeds corresponding to frequencies less than 47.5 Hz or above 52.5 Hz, then the blades of the turbines may get damaged.

- The operation of a transformer below the rated frequency is not desirable. When frequency goes below rated frequency at constant system voltage then the flux in the core increases and then the transformer core goes into the saturation region.

- Loads and other electrical equipment are usually designed to operate at a particular frequency. Off-nominal frequency operation causes electrical loads to deviate from the desired output. The output of power plant auxiliaries like pumps or fans may reduce, causing reduction in power plant output.

##### 1.1.2 LOAD FREQUENCY CONTROL:

If the system is connected to numerous loads in a power system, then the system frequency and speed change with the characteristics of the governor as the load changes. If it’s not required to maintain the frequency constant in a system then the operator is not required to change the setting of the generator. But if constant frequency is required the operator can adjust the velocity of the turbine by changing the characteristics of the governor when required. If a change in load is taken care by two generating stations running parallel then the complex nature of the system increases. The ways of sharing the load by two machines are as follow:

- Suppose there are two generating stations that are connected to each other by tie line. If the change in load is either at A or at B and the generation of A is regulated so as to have constant frequency then this kind of regulation is called as Flat Frequency Regulation.

- The other way of sharing the load is that both A and B would regulate their generations to maintain the frequency constant. This is called parallel frequency regulation.

- The third possibility is that the change in the frequency of a particular area is taken care of by the generator of that area thereby maintain the tie-line loading. This method is known as flat tie- line loading control.

- In Selective Frequency control each system in a group is taken care of the load changes on its own system and does not help the other systems, the group for changes outside its own limits.

- In Tie-line Load-bias control all the power systems in the interconnection aid in regulating frequency regardless of where the frequency change originates.

##### 1.2 MATHEMATICAL MODELLING OF A LFC SYSTEM:

##### 1.2.1 MATHEMATICAL MODELLING OF AGENERATOR:

With the use of swing equation of a synchronous machine to small perturbation, we have

2H ωd2∆δdt2=ΔPm–ΔPe                                                                               (2.1)

Or in terms of small change in speed

d∆ωωsdt=12HsΔPms-ΔPes                                                                           (2.2)

Laplace Transformation gives,

∆Ωs=12HsΔPms-ΔPes                                                                            (2.3)

Fig 2.1: Mathematical modelling block diagram for a generator.

##### MATHEMATICAL MODELLING OFLOAD:

The load on a power system consists of variety of electrical drives. The load speed characteristic of the load is given by:

Δ Pe = ΔPL +D Δω	(2.4)

where ΔPL is the non-frequency sensitive change in load, DΔω is the load change that is frequency sensitive.

D is expressed as percentage change in load divided by percentage change in frequency.

Fig 2.2: Mathematical modelling Block Diagram of Load.

##### 1.2.3MATHEMATICAL MODELLING FOR PRIMEMOVER:

The source of power generation is the prime mover. It can be hydraulic turbines near waterfalls, steam turbine whose energy come from burning of coal, gas and other fuels. The model of turbine relates the changes in mechanical power output ΔPm and the changes in the

steam valve position ΔPV.

GT=∆Pm(s)∆PV(s)=11+ τs.	(2.5)

where the turbine constant is in the range of 0.2 -2.0.

##### 1.2.4 MATHEMATICAL MODELLING FOR GOVERNOR:

When the electrical load is increased suddenly then the electrical power exceeds the input mechanical power. This deficiency of power in the load side is compensated from the kinetic energy of the turbine. Due to this reason the energy that is stored in the machine is decreased and the governor sends signal for supplying volumes of water, steam or gas to increase the speed of the prime mover to compensate deficiency in speed.

∆Pg=∆Pref-1R∆f.                                                   (2.6)

The command ΔPg is transformed through amplifier to the steam valve position command ΔPV. We assume here a linear relationship and considering simple time constant we get this s-domain relation.

∆Pv=11+τs∆Pgs.                                                      (2.7)

Combining all the above block diagrams, for an isolated area system we get the following:

Fig 2.5: complete block diagram of single area system.

The closed loop transfer function that relates the load change∆PLto the frequency deviation ∆Ωis

∆Ω(s)-∆PL=(1+τgs)(1+τTs)(1+τgs)(1+τgs)(2Hs+D)+1R                                                                    (2.8)

##### AUTOMATIC GENERATIONCONTROL:

If the load on the system is suddenly increased, then the speed of the turbine drops before the governor could adjust the input of the steam to this new load. As the change in the value of speed decreases the error signal becomes lesser and the position of the governor and not of the fly balls gets nearer to the point required to keep the speed constant. One way to regain the speed or frequency to its actual value is to add an integrator on its way. The integrator will monitor the average error over a certain period of time and will overcome the offset. Thus, as the load in the system changes continuously the generation is adjusted automatically to restore the frequency to its nominal value. This method is known as automatic generation control. In an interconnected system consisting of several areas, the task of the AGC is to divide the load among the system, stations and generators so to achieve maximum economy and uniform frequency.

##### 1.3.1 AGC IN A SINGLEAREA:

With the main LFC loop, change in the system load will result in a steady state frequency deviation, depending on the speed regulation of the governor. To reduce the frequency deviation to zero we need to provide a reset action by using an PI controller to act on the load reference setting to alter the speed set point. This PI controller would make changes that the final frequency deviation to become zero. The PI controller gain need to be adjusted for obtaining satisfactory transient response.

The closed loop transfer function of the control system is given by:

∆Ω(s)-∆PL=(1+τgs)(1+τTs)(1+τgs)(1+τgs)(2Hs+D)+KPKI+1R     (2.8)

##### FREQUENCY RESPONSES OF LFC SYSTEM:

An isolated power station has the following parameters Turbine time constant = 0.5 s

Governor time constant = 0.2 s Governor inertia constant = 5 s Governor speed regulation = R per unit

The load varies by 0.8 per cent for a 1 per cent change in frequency (D=0.8)

The governor speed regulation is set to R = 0.05 per unit. The turbine rated output is 250 MW at nominal frequency 50 Hz. A sudden load change of 50 MW (𝝙PL= 0.2 Per unit) occurs.

Find steady state Frequency deviation in Hz. Also obtain time domain performance specifications and the frequency deviation step response.

##### FREQUENCY RESPONSE WITHOUT THE PI CONTROLLER:

To obtain the time domain specifications and the step response following command is used: Pl= 0.2; num= [0.1 0.7 1];

den= [1 7.08 10.56 20.8];

t= 0:.02:10;

c= -pl* step (num, den, t);

plot (t, c), xlabel (‘t, sec’), ylabel(‘pu’);title (‘Frequency deviation step response’);

grid timespec (num, den);

Fig 1.4.1Frequency deviation step response without using PI controller.

The time domain specifications are:

Peak time=1.223	Percentage overshoot=54.80

Rise time= 0.418

Settling time= 6.8

The Simulink model for the above system is:

Fig 1.4.2 Simulation Block Diagram of the system without using PI controller.

##### FREQUENCY RESPONSE WITH THE PI CONTROLLER:

Substituting the system parameters, we get the closed loop transfer function as: T(s)= (0.1s3+ 0.7s2+ s)/ (s4+ 7.08s3+ 10.56s2+ 20.8s+ 7)

To find the step response following command is used:

pl= 0.2;

ki= 7;

num= [0.1 0.7 1 0];

den= [1 7.08 10.56 20.8 7];

t= 0:.02:12;

c= -pl* step (num, den, t); plot (t, c), grid

xlabel (‘t, sec’), ylabel(‘pu’) title (‘Frequency deviation step response’)

Fig 1.4.3 Frequency deviation step response with using PI controller.

From the step response we have seen that the steady state frequency deviation is zero, and the frequency returns to its actual value in approximately 10seconds.

\

The Simulink model for the above system is:

Fig 1.4.4 Simulation Block Diagram of the system with using PI controller.

The above discussion shows the load frequency control of a single area power system. The MATLAB results gives the frequency responses of the load frequency control of a single area power system with proportional-integral controller and without proportional-integral controller. In the forthcoming chapters, the Electric Vehicle to Grid Technology integrated with Load frequency control systems and its mathematical modeling will be discussed.

1.5  CONCLUSION.

The above discussion shows the load frequency control of a single area power system. The MATLAB results gives the frequency responses of the load frequency control of a single area power system with proportional-integral controller and without proportional-integral controller.

Chapter 2:  Electric Vehicle to Grid Technology

2.1.  INTRODUCTION:

In response to the need for the additional power source in emergency situations, it is expected that large fleets of electric vehicles (EVs) will constitute an important share of the electricity demand. This evolution is likely to be accompanied by a parallel evolution of the electricity supply with the deployment of smart grid technologies. As a consequence, it is expected that demand will feature higher potential for communication and control, which will enable its active participation in the daily operational planning of power systems. In particular, EVs being equipped with a battery can both defer their demand or inject electricity back into the system. However, to achieve volumes that can have an impact on the system, these demands need to be aggregated and operated as an ensemble.  This permits the scheduling of EV charging and services in coordination with the system operator thus enhancing the power system's efficiency and security while reducing its environmental impact.

2.2. CONCEPT OF ELECTRIC VEHICLE TO GRID TECHNOLOGY:

The vehicle-to-grid (V2G) concept aims to optimize the way we transport, use and produce electricity by turning electric cars into ‘virtual power plants’. Under this relatively new concept, electric cars would store and dispatch electrical energy stored in networked vehicle batteries which together act as one collective battery fleet for ‘peak shaving’ (sending power back to the grid when demand is high) and ‘valley filling’ (charging at  when needed).

V2G would allow consumers to charge electric vehicles and monitor their energy costs, using mobile devices. This information helps utilities to better manage grid loads during peak times.  The information on the car’s state of charge, the vehicle location and the type of power source are managed by the Electric Vehicle Aggregator. Collected data is sent to the PI controller, depending on the grid load, power is extracted from Electric Vehicle.

When the electric utility would like to buy power from the V2G network, it holds an auction. The car owners or leasing companies would be able to define the parameters under which they will sell energy from their battery pack.

## BALANCING TO GRID

There are peak times and off-peak times for energy use, just like there’s rush hour traffic in the morning and in the evening.

Balancing the grid is about making sure there’s enough electricity on the network when we need it in peak times, so the country doesn’t black out. And it also stops power surges, which could damage domestic appliances, or cause electrical fires.

As mentioned earlier, more renewable energy sources are being used to create electricity for the grid. This is a key part of the nation’s move towards a zero carbon future, and a cleaner, greener country for us all. But it makes balancing the grid more tricky – as solar and wind can’t be produced on demand in the same way as fossil fuel energy can.

With technology like EV smart charging, it is far more in control when the vehicle is  charging.  And with V2G, the vehicle owner can also make money from feeding energy back into the grid, to “balance” it in times of need.

2.3.0 Electric Vehicle:

EVs can be roughly divided into two big classes PEVs (plug-in EVs) and non-plug-in EVs, which are essentially HEVs (hybrid EVs). Plug-in electric vehicles (PEVs) use electricity from the energy grid to charge large battery packs, then use the batteries to power an electric motor. In addition, PEVs can be further split into PHEVs (plug-in hybrid EVs) and BEVs (battery EVs). While BEVs are characterized by the use of batteries to supply their electric propulsion system, PHEVs are generally equipped with smaller batteries because they employ an internal combustion engine; this can directly propel the vehicle and/or recharge the battery on board, resulting in an extended mobility range compared to BEVs. EVs could be utilized in a wide range of applications such as smoothing of renewable energy sources and frequency regulation using vehicle-to-grid techniques. Fast power output of EV batteries and thereby the fast response characteristic provides the opportunity to introduce Electric Vehicle to Grid Technology. EVs can be used as generators or loads and hence reduce generation/demand fluctuations, and improve frequency response. For practical participation of EVs in frequency regulation market, a new entity called as EVs aggregator is required to aggregate and control large number of EVs in order to satisfy the frequency regulation criteria. EVs could be effectively used for not only frequency regulation but also for improving stability of smart grid systems when inevitable communication time delays are observed. Furthermore, the dynamic model of an EV battery system is usually first-order transfer function as follows:

GEV=∆PEVUCs=KEV1+sTEV

where, KEV and TEV represent gain and time constant of the EVs battery system, respectively. The turbine of the generator from Generation station and the EVs aggregator have participation ratios of α0 and α1 which are allocated to determine their participation in frequency regulation service. For example, α1 =1 means that the turbine is totally employed to regulate the system frequency. Note that the total amount of α0 and α1 is equal to one and is also total contribution of both generation units (Grid and Electric Vehicle) for frequency regulation service.

2.4.0 Why do we use Plug in Electric vehicle for V2G technology?

Since  they are usually powered  by  batteries that  can  store it is easier to  deliver energy to  the  grid when needed. This way, PEVs can work as a distributed generators and provide different  services  when they are  parked and connected to an   electrical   outlet. Exploiting   these services add an extra economic value to PEVs, making them more profitable for PEV owners and  accelerating their market. Also, due to the fact that PEV batteries charging period at rated power generally requires much less time than plug-in parking periods, it is possible to optimize the PEV charging process from the economical point of view . However, PEV batteries can also be employed to reduce negative impacts of PEVs on power systems, even increasing their performances.

2.5.0 What is V2G charging or EV smart charging?

V2G charging refers to the 2-way flow of electrical energy from the grid, through a specially-built charger into an EV, and back again. As an EV owner, one can choose to charge the car up from the grid – or sell stored energy in the car back to the grid.

2.6.0 Electric Vehicle Aggregator:

The massive deployment of the EV and interaction with the power system (i.e., transmission and distribution system operators) requires an agent responsible for aggregating EV and managing their charging process. This agent is called aggregator and it is an enabler of the EV integration in the electricity market and power system operations. In other words, Electric vehicles aggregator is  an intermediate between electric vehicle (EV) and operator of power grid. The EVs aggregator is responsible for the management of EVs in order to supply the owners with their orders and also for maximizing the profits of power grid in electricity market.

The main function of EV aggregator is to send and receive information regarding the charging status of EV’s in order to adjust their power using an automatic generation control in V2G technology.

2.6.1 AN EV AGGREGATOR MODEL:

An EV aggregator model consisting of a large number of EVs, each of which has a delay component. The dynamic model of an EV battery system is usually described as the following first-order transfer function:

GEV=∆PEVUCs=KEV1+sTEV

where KEV denotes the gain of the EV and TEV denotes the time constant of the EV battery system. The delay taken for receiving control signals from the EV aggregator is modeled as an exponential transfer function of e-sτ(t), where τt denotes a time-varying delay function which models the communication delay from the EV aggregator to the EV and the scheduling delay in the EV aggregator.

For analytical simplicity, we assume that the delays for all EVs are the same as Tdelay on an average sense and the time constants TEV of all EVs are the same. From the assumption, we obtain an aggregated EV aggregator model consisting of one delay function and one EV dynamics model.

Fig.2.1: LFC system integrated with an EV aggregator.

2.6.2 LFC Model including EV Aggregators with Delays:

Fig. 1 shows a single-area LFC model including multiple EV aggregators with time-varying delays. Except the model of EV aggregators with time-varying delays, other components have been modeled in many LFC-related studies. We assume that all synchronous generators have reheat thermal turbines. We adopt, as an LFC controller, a PI-type controller which has been used for LFC in Germany, Netherlands, Belgium, etc. The output of the PI controller is distributed to the reheat thermal generator and N EV aggregators depending on the participation ratiosα0, α1,….,αN, where α0 denotes the participation ratio of an aggregated thermal generator andαi denotes the participation ratio of EV aggregator i for i = 1……N.

The linearized dynamic model of the single-area LFC model including N EV aggregators with time-varying delays can be expressed as follows:

(2.1)

xt=Axt+B0ut+k=1NBku(t-τkt+F∆Pd

yt=Cxt

Where

xt=[∆f  ∆Pg∆Pm∆Xg∆PEV,1… ∆PEV,N]T

yt=ACE,

B0=[0  0  0  Fpα0Tgα0Tg  0  ⋯  0 ]T

Bk= αkKEV,KTEV,Ke4+k,      (k=1,….,N)

C=[ β 0 0 0 0 ⋯0 ],

F = [ -1M  0  0  0  0  ⋯  0]T

A =  -DM1M001M⋯1M0-1Tc1Tc00⋯0-FpRTg0-1TrTg-FpTrTrTg0⋯0-1RTg00-1Tg0⋯00000-1TEV,1⋯0⋮⋮⋮⋱⋱⋯⋮00000⋯-1TEV,N

And  ∆f,∆Xg,∆Pm and ∆Pg denote the derivation of the frequency, valve position, mechanical power output and generator power output, respectively.∆PEV,1(k=1,….N) is the derivation of the power output in the k-th EV aggregator.

The ACE signal is defined as follows:

ACE=β∆f.

A PI-type LFC controller is designed as follows:

ut= -KpACE-KIACEdt.                                           (2.2)

Fig 2.2 Electric Vehicle to Grid Technology

Fig.2.2 shows the block diagram of Electric vehicle to grid technology. The load disturbance is sensed using frequency sensor. Whenever there is heavy power demand, the frequency of voltage of the tie lines connecting the consumer and the grid reduces. This is reduction in frequency is sensed by frequency sensor. The load disturbance sensed by frequency sensor is feedback to Proportional-Integral Controller. The PI controller in turn decides how much power to be extracted from EV aggregator along with the Gird to compensate the power requirements. According to PI controller’s instruction, the governor valve is opened to let the necessary amount of steam to generate more power. The turbine takes input(steam) from governor and does its work. The generator in turn takes the output of turbine as its input and converts into electrical energy. Meanwhile, the amount of power to be extracted from EV aggregator also comes into picture. The power generated from the Grid and the power extracted from the EV aggregator is now distributed to load.

In this chapter, Electric vehicle to grid technology and EV aggregator model are discussed. In next chapter mathematical modeling of Load Frequency Control System integrated with EV aggregator will be discussed.

CHAPTER-3

MATHEMATICAL MODELLING OF LOAD FREQUENCY CONTROL SYSTEMS INTEGRATED WITH EV AGGREGATOR

3.1.0 INTRODUCTION:

The design and implementation of load frequency control systems integrated with Electric Vehicle Aggregator need to be supported by a holistic understanding of their functional processes, their interactions, and their responses to various changes. Models developed to represent different functional processes and the systems are seen as useful tools to support the related studies for different stakeholders in a tangible way. This chapter presents an overview of modeling approaches applied to support aggregation of EVs with Load Frequency Control System. As already known, in the integrated electrical systems, frequency control service considering the Electric Vehicle (EV) aggregators could lead to time variable delay in Load Frequency Control Schemes. Thus, this chapter illustrates different time-variable delays result based on the stability of an LFC system in the presence of the EV aggregators. Primarily, a graphical method characterizing the stability boundary locus is implemented. For a given time delay, the method computes all the stabilizing proportional-integral (PI) controller gains, which constitutes a stability region in the parameter space of PI controller. Secondly, in order to complement the stability regions a frequency-domain exact method is used to calculate stability delay margins for various values of PI controller gains. The impact of EV aggregator on both stability regions and stability delay margins is thoroughly analyzed and the results are authenticated by the usage of visual programming (MATLAB) and time domain simulation.

3.1.1 MATHEMATICAL MODELLING OF LFC SYSTEM WITH EV AGGREGATOR:

The block diagram of a single area LFC system including an EV aggregator and delay block is presented in Fig.1

Fig 3.1

The PI type controller is adopted as LFC controller. In Fig.1, Δf,ΔXg,ΔPm, ΔPEV and ΔPd are the deviation of frequency, valve position, mechanical power output, generator power output, EV aggregator power output and load disturbance, respectively; D,M,R,β,Fp,Tg,Tr and Tc are the damping coefficient, generator inertia constant, speed drop,  frequency bias factor, fraction of the total turbine power, time constant of the governor, reheat and turbine, respectively; and KP and KI are the PI controller gains.

The imbalance between demand and generation is measured in terms of incremental frequency variable Δf. The incremental variable is fed back to PI controller which sends appropriate control effect to the governor. The governor decides the valve opening of the turbine for increasing input of the synchronous generator. The constant action restores the imbalance between generator and demand. The fleet of Electric Vehicles called Ev aggregator connected through communication network and the conventional power system restores the frequency imbalance brought about by load variation. The load sharing between the conventional power system and EV aggregator is taken care using participation factors α0 and α1.

3.1.2  VARIABLE DESCRIPTION:

3.1.3 PARAMETER DESCRIPTION:

3.1.4 CONTROLLER PARAMETERS AND PARTICIPATION FACTORS:

3.1.5 EV AGGREGATOR PARAMETERS:

For mathematical modeling purpose, the LFC system is illustrated as Fig.3.2,

Let us consider,

(3.1)

G1=1(D+SM)

H1= -[s[kp βα0R+1]  +kIβα0R][1+SFpTr]Rs1+STg1+STc(1+STr)

H2=-[kp βα1kev s+k2βα1kev ]e-sτs+s2TEV

M=8.8; D=1 ; β=1 ; Fp=16 ;

TEV=0.1 ;Tg =0.2 ;   Tc=0.3 ; Tr=12  ;

Kev =1 ;Kp =0.2 ;  KI =0.6 ;

α0 =0.8 ;  α1 =0.2 .

By using signal flow graph method, we find the transfer function of the System model of single-area LFC with EV aggregator.

P1=G1;   ∆1=1-0=1.

L1=G1H1 .         ;          L2=G1H2  ;      ∆ =1-H1+H2G1.

Maison’s Gain formula:

Transfer function=P1∆1∆.

= G1  ×  11-G1[H1+H2]

= 1D+SM1+1D+SM[Skp βα0R+1 +kIβα0R1+SFpTrRs1+STg1+STc1+STr+-kp βα1kev s+k2βα1kev e-sτs+s2TEV]

=1D+SMRs1+STg1+STr1+STCD+SMs+s2TEV]+[skp βα0R+1 +kIβα0R1+SFpTrs+s2TEV+Rs1+STg1+STc(1+STr)-[kp βα1kev s+k2βα1kev ]e-sτD+SM1+STg1+STc1+STr(s+s2TEV)RS

The closed loop transfer function of the time-delayed load frequency control system relates Δf(s), the incremental frequency variable (output variable) to load disturbance variable ΔPd(s) (input variable).

The closed loop transfer function is derived as follows:

Gs=∆fs∆Pds=NsPs+Qse-sτ

N(s) =n5s5+n4s4+n3s3+n2s2+n1s1+n0

=s5[RTEVTRTcTg]+s4[R[TgTRTEV+TRTcTEV+TcTgTEV+TRTcTg]+ s3[R[TgTEV +TRTEV+TcTEV+TRTg+TRTc+TcTg]]+s2 [R[TR+Tg+ Tc+TEV]] +s[R]

P(s) =p6s6+ p5s5+p4s4p3s3+p2s2+p1s1+p0

=s6MRTgTrTcTEV+s5DRTgTrTcTEV + MR (TgTrTc+TrTcTEV+TgTcTEV+   TgTrTEV)+s4DR(TgTrTc+TrTcTEV+Tg+TgTrTEV)+MR(TrTc+TgTc+TgTr+TgTEV+TrTEV+TcTEV)+ s3DR(TrTc+TgTc+TgTr+TgTEV+ TrTEV+TcTEV)+MRTr+Tc+Tg+TEV+FpTrTEV+α0βRKpFpTrTEV+s2DRTr+Tc+   Tg+TEV+MR+FpTr+TEV+α0βRKpTEV+KpTrFp+   K1FpTrTEV + s1DR+1+  α0βR(Kp+k1TEV+k1TrFp +  α0βRK1

Q(s) =          q4s4+q3s3+qs2+q1s1+q0

=     s4α1βRKEVKpTgTrTc + s3α1βRKEVKpTrTc+KpTgTc+                      KpTgTr+K1TgTrTc  +  s2α1βRKEVKpTc+KpTg+KpTr+                      K1TrTc+K1TgTc+K1TrTg  + s1α1βRKEVKp+K1Tc+K1Tr+K1Tg                                    +α1βRKEVK1

For stability region and delay margin computations, it is necessary to obtain the characteristic equation of the single area LFC-EV system.

The characteristic equation of the single area LFC-EV system:

∆(s,τ)=Ps+Q(s)e-sτ

∆(s,τ)=s6MRTgTrTcTEV  + s5DRTgTrTcTEV + MR (TgTrTc+     TrTcTEV+                         TgTcTEV+   TgTrTEV)+s4DR(TgTrTc+TrTcTEV+TgTcTEV+                         TgTrTEV)+MR(TrTc+TgTc+TgTr+TgTEV+TrTEV+TcTEV)  + s3DR(TrTc+TgTc+TgTr+TgTEV+ TrTEV+TcTEV)+MRTr+Tc+Tg+TEV+FpTrTEV+α0βRKpFpTrTEV+s2DRTr+Tc+  TEV+MR+FpTr+TEV+α0βRKpTEV+KpTrFp+     K1FpTrTEV + s1DR+1+  α0βR(Kp+k1TEV+k1TrFp +                α0βRK1    +    s4α1βRKEVKpTgTrTc + s3α1βRKEVKpTrTc+ KpTgTc+ KpTgTr+K1TgTrTc  +  s2α1βRKEVKpTc+KpTg+   KpTr+TrTc+K1TgTc+K1TrTg  + s1α1βRKEVKp+K1Tc+K1Tr+K1Tg   +  α1βRKEVK1e-sτ.

The roots of the characteristic equation are:

0.05765s61.06734s5+6.02351s411.6329s3+7.20563s2+6.2595s1+1.145 = 0

S =   -9.90431

S =   -5.36025

S =   -2.84345

S =   -0.221403

S =   -0.0923932 ±0.765317i

The necessary condition for the single-area LFC-EV system to be asymptotically stable is that all the roots must be in the left half of the s-plane. In consideration of the single delay, the delay margin computation can be done by finding values of τ* for which has roots on the jω-axis. For some finite value of τ*, the characteristic polynomial of Δ (s, τ*) =0 has a root on the imaginary axis at s= jωc, the equation of Δ (-s,τ*) =0 will also have the same root on the imaginary axis for the same value of τ* and ωcdue to the complex conjugate symmetry of complex roots. That means s= jωc will be a common root of the following equation:

∆(jωc,,τ*)  =  Pjωc+Q(jωc)e-jwcτ* =0

∆-jωc,τ*= P-jωc+Q(-jωc)ejwcτ* =0

By eliminating the exponential terms between the above two sub-equations, the following augmented polynomial is obtained:

W(ωc2)= P (jωc)P (-jωc)-Q(jωc) Q(-jωc)=0

P(s)=p6s6+p5s5+p4s4+p3s3+p2s2+p1s1+p0.

(1)

P(-s) =p6s6-p5s5-p4s4-p3s3-p2s2-p1s1-p0.

P(s)×P(-s)

=   s12p6p6-s11p5p6++s10p4p6-s9p3p6+s8p2p6-s7p1p6+s6p6p0+s11p5p6+s10p5p5+s9p5p4-s8p5p3+s7p5p2-s6p5p1+s10p6p4-s9p5p4+s8p4p4-s7p3p4+s6p2p4-s5p1p4+s4p0p4+s9p3p6-s8p3p5+s7p3p4-s5p3p2-s4p3p1+s3p3p0+s8p6p2-s7p5p2+s6p4p2-s5p3p2-s4p2p2-s3p1p2+s2p0p2+s7p6p1-s6p5p1+s5p4p1-s4p3p1+s3p2p1-s2p1p1+s1p0p1+s6p6p0-s5p5p0+s4p4p0-s3p3p0+s2p2p0-s1p0p1+p02.

P(s)×P(-s) =s12p6p6-s10(p52-2p4p6)+s8(p42+2p2p6-2p5p3)-s6(p32+2p2p4+2p5p1-2p6p0)+s4(p22-2p3p1+2p4p0)-s2(p12-2p2p0)+p02.

(2)

Q(s) =q4s4+q3s3+q2s2+q1s1+q0.

Q(-s) =q4s4-q3s3-q2s2-q1s1-q0.

Q(s)×Q(-s)=s8q42-s7q4q3+s6q4p2-s5q4q1+s4q3q0+s7q3q4-s6q3q3+s5q3q2-s4q3q1+s3q3p0+s6q2q4-s5q3q2-s4q2q2-s3q2q1+s2q2q0+s5q1q4-s4q3q1+s3q2q1+s2q2q0-s1q1q0+q02.

Q(s)×Q(-s) =s8q42-s6(q32-2q4q2)+s4(q22+2q3q0+2q3q1)-s2

(q12-2q2q0)+q02.

Substitute s = jωc,

P(jω)×P(-jω)-Q(jω)×Q(-jω)  =  ωc12p62-ωc10(p5 2-2p6p4)+ωc8(p42+2p2p6-2p5p3)-ωc6(p32+2p2p4+2p5p1-2p6p0)+ωc4 (p22- 2p3p1+2p4p0)-ωc2(p12-2p2p0)+p02 - ωc8q42+ωc6q32-2q4q2-ωc4(q22+2q3q0-2q3q1) +ωc2(q12-2q2q0) - q02

By substituting the polynomials of P(jωc), P(-jωc),Q(jωc)and Q(-jωc)into the augmented polynomialW(ωc2) of can be represented as

W(ωc2)= t12ωc12+ t10ωc10+t8ωc8+t6ωc6+t4ωc4+t2ωc2+t0=0.

t12=p62 .

t10=p52- 2p6p4 .

t8=p42+ 2p6p2-2p5p3–q42.

t6=p32-2p6p0-2p4p2+2p5p1-2q4q2- q32.

t4=p22 + 2p4p0-2p3p1- 2q4q0+2q3q1-q22.
t2=p12-2p2p0+2q2q0-q12.

t0=p02- q02.

p6=0.05765	;                    q4=0.05497;                    t12= 0.003323

p5 =1.06734;                   q3=0.62764;                      t10=0.4510

p4=5.96854;                 q2=    2.33573;                  t8=12.6955

p3= 11.0053 ;                  q1=2.9395;                       t6=69.828

p2  = 4.8699;                      q0=    0.2290;                    t4=-40.217

p1= 3.320;                         t2=-5.470

p0= 0.916;                          t0=0.78661

W (ωc2) = 0

W (ωc2) =0.00323ωc2+0.451ωc2+12.6955ωc2+69.828ωc2-40.217ωc2 - 5.47ωc2+0.786615=0

ωc2= −103.77

ωc2 = −28.3095

ωc2 = −8.06205

ωc2 = −0.184679

ωc2 = 0.091843

ωc2 = 0.606242

The real positive roots are ωc = 0.3030561 and ωc = 0.7786154. Calculating the delay margin for each positive root and the minimum of those is taken as the system delay marginτ*.

The stability delay margin is τ*=0.6018600127

COMPUTATION OF STABILITY REGIONS

To identify the boundary of the stability region in the parameter space of PI controller, (Kp-KI)-plane for a given time delay τ, s = jωc and the crossing frequency ωc> 0 is substituted into (2). The PI controller gains are then separated to obtain a new equation as follows

P(s) =         p6s6+ p5s5+   p4s4+p3s3+ p2s2+  p1s1 +   p0

Pjωc=p6ωc6 + p5jωc5+p4ωc4-p3jωc3– p2ωc2+p1ωc+p0

Pjωc= [- p6ωc6+ p4ωc4– p2ωc2+p0] +j[p5ωc5-p3jωc3+p1ωc}

Q(s) =          q4s4+q3s3+qs2+q1s1+q0

Qjωc=q4ωc4-q3jωc3– q2ωc2+q1ωc+q0

Qjωc=[q4ωc4– q2ωc2+q0] +j [q3ωc3+q1ωc]

e-st =e-jωct=cosωct-j sinωct

Pjωc+Qjωce-jωct=0

[q4ωc4– q2ωc2+q0] +j[q3ωc3+q1ωc] * cosωct-j sinωct=-Pjωc

[q4ωc4– q2ωc2+q0] cosωct-[jq4ωc4– q2ωc2+q0] sinωct+ j[-q3ωc3+q1ωc] *-j sinωct+ j[-q3ωc3+q1ωc]cosωct=

-[ Pjωc R + j PjωcQ]

q4ωc4– q2ωc2+q0] cosωct+[-q3ωc3+q1ωc] sinωct =PjωcR

q4ωc4– q2ωc2+q0] cosωct+[-q3ωc3+q1ωc] sinωct =-PjωcR

q4ωc4– q2ωc2+q0] cosωct-[-q3ωc3+q1ωc] sinωct =-PjωcI

q4,= Kpα1βRKEVTgTrTc

q3, =  α1βRKEVKpTrTc+KpTgTc+KpTgTr

q3,, =KIα1βRKEVTgTrTc

q2, =  α1βRKEVKpTc+KpTg+KpTr

q2,, =KIα1βRKEVTrTc+TgTc+TgTr

q1,=α1βRKEVKp

q1,,=KIα1βRKEVTgTrTc

q0,,=KIα1βRKEVKI

p6=MRTgTrTcTEV

p5=DRTgTrTcTEV + MR (TgTrTc+TrTcTEV+ TgTcTEV+ TgTrTEV)

p4=DR(TgTrTc+TrTcTEV+TgTcTEV+TgTrTEV)+MR(TrTc+TgTc+TgTr+TgTEV+TrTEV+TcTEV)

p3=DR(TrTc+TgTc+TgTr+TgTEV+ TrTEV+TcTEV)+MRTr+Tc+Tg+TEV+FpTrTEV

p3,= Kp[α0βRKpFpTrTEV];p2=DRTr+Tc+Tg+TEV+MR+FpTr+TEV

p2, =Kp[α0βRFpTr+TEV];p2,,=KI[α0βRFpTrTEV]

p1 = DR+1

p1, = Kp[α0βR[+1]]

p1,, =KI[α0βRFpTr+TEV]

p0,, = Kp[α0βR]

q4ωc4– q2ωc2+q0] cosωct+[-q3ωc3+q1ωc] sinωct= -[- p6ωc6+ p4ωc4– p2ωc2+p0].

[-q3ωc3+q1ωc] cosωct - q+4ωc4– q2ωc2+q0] sinωct =- [p5ωc5-p3jωc3+p1ωc]

REAL PART

kp[q4,ωc4 - q2,ωc2cos(ωc+1)  -q3,ωc3- q1,ωcsin(ωc+1)]

kI[-q2,,ωc2+q0,,cos(ωc+1)  -q3,,ωc3- q1,,ωcsin(ωc+1)]

=  [- p6ωc6+ p4ωc4– p2ωc2kp[-p2,ωc2] kI[-p2,,ωc2+p0,,.

kp[-p2,ωc2+q4,ωc4 cos(ωc+1) –q2,ωc2cos(ωc+1) –q3,ωc3sin(ωc+1) +q1'ωcsin(ωc+1)] + kI[-p2''ωc2+p0''-q2''ωc2 cos(ωc+1) + q0''cos(ωc+1) – q3''ωc3sin(ωc+1) +q1''ωcsin(ωc+1)] +[ p6ωc6– p4ωc4+p2ωc2].kpA1ωc+ kIB1ωc =  -C1ωc

IMAGINARY PART

kp-q3,ωc4cos(ωct)  + q4,ω43+q2,ωc2sin(ωct)]+kI[-q3,,ωc2cos(ωct)  +q1,,ωc- q1,,ωccos(ωct)   +    -q2,,ωc2- q0,,sin(ωct)]

=-[ p5ωc5-p3jωc3+p1ωckp[-p3,ωc3+p1,ωc3] kI[p1,,ωc]

kp[-p3,ωc3+p1,ωc-q3''ωc3 cos(ωct) +q4'ωc4sin(ωct) +q2'ωc2sin(ωct) + kI[p1''ωc-q3''ωc3 cosωct+q1''ωc2 cos(ωct) +q2''ωc2sin(ωct) - q0''ωcsin(ωct)]

=[ -p5ωc5– p3ωc3-p1ωc ]

(3)

kpA2ωc+ kIB2ωc =  -C2ωc

(4)

kpA2ωc+ kIB2ωc + C2ωc = 0

From 3 and 4 we get

kp= B1ωcC2ωc-B2ωcC1ωcA1ωcB2ωc-A2ωcB1ωc→(5)

From 3 and 4 we get

kp= A2ωcC1ωc-A1ωcC2ωcA1ωcB2ωc-A2ωcB1ωc→(6)

(7)

KpA1(ωc)+ KIB1(ωc)+ C1 (ωc)=0

KpA2(ωc)+ KIB2(ωc)+ C2 (ωc)=0

The coefficients are

A1 (ωc) = -pי2ωc2 + qי4ωc4cos(ωct) – qי2ωc2cos(ωct) – qי3ωc3sin(ωct) + qי1ωcsin(ωct)

B1(ωc) = -pיי2ωc2 + pיי0 – qיי2ωc2cos(ωct) +qיי0cos(ωct) – qיי3ωc3sin(ωct) + qיי1ωcsin(ωct)

C1 (ωc) = -pωc6 + pωc4 – p2ωc2

A2(ωc) = -pי3ωc3 + pי1ωc – qי3ωc3cos (ωct) + qי4ωc4sin(ωct) + qי2ωc2sin(ωct)

B2(ωc) = pיי1ωc– qיי3ωc3cos(ωct) + qיי1ωccos(ωct) + qיי2ωc2sin(ωct) – qיי0sin(ωct)

C2(ωc) = p5ωc5 – p3ωc3 + p1ωc

It should be noted that p6, p5 and p4 coefficients in the above equations are those given in (1) and q3, q4 and q3 coefficients in the above equations are those given in (2) on the other hand the coefficients of pיי and qיי in the above equations corresponds to the remaining terms of p and q containing KI, respectively. The stability boundary obtained by (10) is called as complex root boundaries (CRBs) of the LFC-EV system. It is noted that a real root may cross the jω axis origin. Moreover, it can be observed that such a stability change occurs only for  KI=0, defining another boundary called as Real Root Boundary (RRB) locus. As a result, the (KP, KI)-plane is divided into stable and unstable regions by the RRB locus KI =0 and the CRB locus, obtained by (7) which will be discussed next chapter.              CHAPTER 4:   SIMULATIONS ANDRESULTS

4.1.0  DELAY DEPENDENT STABILITY:

Computation of stability delay margin:

The objective is to compute the maximum value of the delay ( τ*) within which the closed loop system remains asymptotically stable. In this problem, all the load frequency control system parameters are known and The PI controller parameters Kp and KI  are assumed to be known.

Simulink Model of Single Area LFC control system with EV aggregator:

Fig 4.1 Simulink Model of Single Area LFC control system with EV aggregator.

4.1.1 BENCHMARK SYSTEM PARAMETERS:

| SL.NO | PARAMETERS | DESCRIPTION | VALUE |
| --- | --- | --- | --- |
| 1. | M | Generator inertia constant | 8.8 |
| 2. | D | Damping Coefficient | 1 |
| 3. | Tg | Governor Time Constant | 0.2 |
| 4. | TC | Generator Time Constant | 0.3 |
| 5. | Tr | Reheat and Turbine Time Constant | 12 |
| 6. | FP | Fraction of total turbine power | 1/6 |
| 7. | R | Speed drop | 1/11 |
| 8. | β | Frequency bias factor | 21 |
| 9. | KEV | Gain of EV aggregator | 1 |
| 10. | TEV | Time Constant of EV aggregator | 0.1 |

Table 4.1 Parameters of LFC system.

These are the benchmark parameters used to compute the maximum value of the delay ( τ*) within which the closed loop system remains asymptotically stable.

The MATLAB program to compute the stability delay margin is given below,

4.1.2 MATLAB code for computation of stability delay margin:

clear ALL

M=    8.8;

D=    1;

Tg=   0.2;

Tc=   0.3;

Tr=   12;

Fp=   1/6;

R=    1/11;

Beta= 21;

Kev= 1;

Tev= 0.1;

alpha0= 0.8;

alpha1= 0.2;

Ki = 0.6;

Kp = 0.2;

p6= (M*R*Tg*Tr*Tc*Tev);

p5= (D*R*Tg*Tr*Tc*Tev) + (M*R) *(Tg*Tr*Tc + Tr*Tc*Tev + Tg*Tc*Tev + Tg*Tr*Tev);

p4= (D*R) *(Tg*Tr*Tc + Tr*Tc*Tev + Tg*Tc*Tev + Tg*Tr*Tev) + (M*R) *(Tr*Tc + Tg*Tc + Tg*Tr + Tc*Tev + Tg*Tev+ Tr*Tev);

p3= (D*R) *(Tc*Tr+ Tg*Tc + Tg*Tc + Tg*Tr + Tc*Tev + Tr*Tev + Tg*Tev) + (M*R) *(Tc+Tr+Tg+Tev) +(Fp*Tr*Tev) + (alpha0*Beta*R*Kp*Fp*Tr*Tev);

p2= (D*R) *(Tc+Tr+Tg+Tev) +(M*R) +(Fp*Tr) +Tev+(alpha0*Beta*R) *(Kp*Tev + Kp*Fp*Tr + Ki*Fp*Tr*Tev);

p1= (D*R) + 1 + (alpha0*Beta*R) *(Kp+ Ki*Tev + Ki*Fp*Tr);

p0= (alpha0*Beta*R*Ki);

q4= (alpha1*Beta*R*Kev*Kp*Tg*Tr*Tc);

q3= (alpha1*Beta*R*Kev) *(Kp*Tr*Tc + Kp*Tg*Tc + Kp*Tg*Tr + Ki*Tg*Tc*Tr);

q2= (alpha1*Beta*R*Kev) *(Kp*Tc + Kp*Tr + Kp*Tg + Ki*Tr*Tc + Ki*Tg*Tc +Ki*Tg*Tr);

q1= (alpha1*Beta*R*Kev) *(Kp + Ki*Tc + Ki*Tr + Ki*Tg);

q0= alpha1*Beta*R*Kev*Ki;

t12= p6^2;

t10= p5^2 - 2*p6*p4;

t8 = p4^2 + 2*p6*p2 - 2*p5*p3 - q4^2;

t6 = p3^2 - 2*p6*p0 - 2*p4*p2 + 2*p5*p1 + 2*q4*q2 - q3^2;

t4 = p2^2 + 2*p4*p0 - 2*p3*p1 - 2*q4*q0 + 2*q3*q1 - q2^2;

t2 = p1^2 - 2*p2*p0 + 2*q2*q0 - q1^2;

t0 = p0^2 - q0^2;

k=1;

r = roots ([t12 t10 t8 t6 t4 t2 t0]);

for i=1: size(r,1)

an(i)=angle(r(i)) *(180/pi);

if (an(i)==0)

w1(k)=r(i);

k=k+1;

end

end

m = 0;

for n = 1: size(w1,2)

wa (: n) = sqrt (w1(: n));

wc = wa (: n);

derWc = 6*t12*wc^10 + 5*t10*wc^8 + 4*t8*wc^6 + 3*t6*wc^4 + 2*t4*wc^2 + t2;

RT = sign(derWc);

if(RT==1)

m = m+1;

P = p6*(j*wc) ^6 + p5*(j*wc) ^5 + p4*(j*wc) ^4 + p3*(j*wc) ^3 + p2*(j*wc) ^2 + p1*(j*wc) ^1 + p0;

Q = q4*(j*wc) ^4 + q3*(j*wc) ^3 + q2*(j*wc) ^2 + q1*(j*wc) ^1 + q0;

SIN = imag(P/Q);

COS = real(-P/Q);

theta = atan(SIN/COS);

if COS<0

tau1(:m) = ((theta+pi)/wc);

else

tau1(:m) = (theta/wc);

end

RT;

wc;

tau1;

else

RT;

wc;

end

end

Kp

Ki

format long g

tau = min(tau1)

format

The result of the above-mentioned MATLAB code is:

4.1.3 MATLAB output for different Kpand KIvalues:

τ*   -  STABILITY DELAY MARGIN (s)

| Kp | KI=0.2 | KI=0.4 | KI=0.6 | KI=0.8 | KI=1.0 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1.7866 | 0.4231 | 0.0859 | -0.0615 | -0.1417 |
| 0.1 | 2.7421 | 0.8271 | 0.3496 | 0.1340 | 0.0134 |
| 0.2 | 3.5861 | 1.2002 | 0.6018 | 0.3242 | 0.1656 |
| 0.3 | 4.2522 | 1.5214 | 0.8327 | 0.5034 | 0.3115 |
| 0.4 | 4.6970 | 1.7773 | 1.0346 | 0.6669 | 0.4477 |
| 0.5 | 4.8537 | 1.9635 | 1.2023 | 0.8108 | 0.5717 |
| 0.6 | 4.6817 | 2.0829 | 1.3339 | 0.9327 | 0.6812 |
| 0.7 | 4.3135 | 2.1433 | 1.4300 | 1.0317 | 0.7751 |
| 0.8 | 3.9050 | 2.1545 | 1.4932 | 1.1080 | 0.8527 |
| 0.9 | 3.5197 | 2.1273 | 1.5275 | 1.1628 | 0.9142 |
| 1.0 | 3.1740 | 2.0715 | 1.5372 | 1.1982 | 0.9606 |

Table 4.2 stability delay margin for various Kpand KIvalues.

4.1.4 SIMULATION RESULTS:

Stability analysis for various communication delays. This simulation results shows the impact of communication delay in the load frequency control system integrated with electric vehicle aggregator.

1.For Kp=0.2,KI=0.6 and Communication Delay, τ*= 0.6018 sec

Marginal stable response:

In this, the frequency response exhibits sustained oscillations which indicate the marginal stability of LFC-EV system.

Fig 4.1 the plot of incremental frequency vs time for τ*= 0.6018 sec

- For Kp=0.2,KI=0.6 and Communication Delay, τ*= 0.5751sec
Stable response:

In this, the frequency response exhibits decaying oscillations which indicate the stability of LFC-EV system.

Fig 4.2 the plot of incremental frequency vs time for τ*= 0.5751sec

3. For Kp=0.2,KI=0.6 and Communication Delay, τ*= 0.6252 sec

Unstable response:

In this, the frequency response exhibits growing oscillations which indicate the instability of LFC-EV system.

Fig 4.3 the plot of incremental frequency vs time for τ*= 0.6252 sec

4.2.0 COMPUTATION OF STABILITY DELAY MARGINS:

The objective is to compute a feasible origin in PI controller parametric space. In this problem, all the LFC system parameters except controller parameters are known. The time-delay in the communication network is assumed to be known.

The MATLAB program to compute the stability delay regions is given below,

4.2.1 MATLAB CODE FOR COMPUTATION OF STABILITY REGIONS:

Clear all

M=8.8;

D=1;

Tg=0.2;

Tc=0.3;

Tr=12;

Fp=1/6;

R=1/11;

beta=21;

Kev=1;

Tev=0.1;

alpha1=0.1;

alpha0=0.9;

Q4=Kp*[alpha1*beta*R*Kev*Tg*Tr*Tc];

Q3= Kp*[alpha1*beta*R*Kev*[Tr*Tc+Tg*Tc+Tg*Tr]];

Qq3=Ki*[alpha1*beta*R*Kev*Tg*Tr*Tc];

Q2=Kp*[alpha1*beta*R*Kev*[Tc+Tr+Tg]];

Qq2=Ki*[alpha1*beta*R*Kev*[Tr*Tc+Tc*Tg*Tc+Tg*Tr];

Qq0=Ki*[alpha1*beta*R*Kev*Ki];

Q1=Kp*[alpha1*beta*R*Kev];

Qq1=Ki*[alpha1*beta*R*Kev*[Tc+Tr+Tg]];

p6=M*R*Tg*Tr*Tc*Tev;

p5=D*R*Tg*Tc*Tr*Tev+M*R*[Tg*Tr*Tc+Tr*Tc*Tev+Tg*Tc*Tev+Tg*Tr*Tev];

p4=D*R*[Tr*Tg*Tc+Tr*Tc*Tev+Tg*Tc*Tev+Tg*Tr*Tev]+M*R*[Tr*Tc+Tg*Tc+Tg*Tr+Tc*Tev+Tr*Tev+Tg*Tev];

p3=D*R*[Tr*Tc+Tg*Tc+Tg*Tr+Tc*Tev+Tr*Tev+Tg*Tev]+M*R*[Tc+Tr+Tg+Tev]+Fp*Tr*Tev;

P3=Kp*[alpha0*beta*R*Fp*Tr*Tev];

p2=D*R*[Tc+Tr+Tg+Tev]+M*R+Fp*Tr+Tev;

P2=Ki*[alpha0*beta*R*[Tev+Fp*Tr]];

Pp2=Ki*[alpha0*beta*R*Fp*Tr*Tev];

p1=D*R+1;

P1=Kp*[alpha0*beta*R+1];

Pp1=Ki*[alpha0*beta*R*[Tev+Fp*Tr]];

Pp0=Ki*[alpha0*beta*R];

t = 0.5;

wc = 0:0.01:3.91;

for i=1:size(wc,2)

A1(:,i) = -P2*wc^2+Q4*wc^2*cos(wc*t)-Q2*wc^2*cos(wc*t)-Q3*wc^3*sin(wc*t)+Q1*wc*sin(wc*t);

B1(:,i) = -Pp2*wc^2+Pp0-Qq2*wc^2*cos(wc*t)+Qq0*cos(wc*t)-Qq3*wc^3*sin(wc*t)+Qq1*wc*sin(wc*t);

A2(:,i) = -P3*wc^3+P1*wc-Q3*wc^3*cos(wc*t)+Q4*wc^4*sin(wc*t)+Q2*wc^2*sin(wc*t);

B2(:,i) = Pp1*wc-Qq3*wc^3*cos(wc*t)+Qq1*wc*cos(wc*t)+Qq2*wc^2*sin(wc*t)-Qq0*sin(wc*t);

C1(:,i) = -p6*wc^6+p4*wc^4-p2*wc^2;

C2(:,i) = p5*wc^5-p3*wc^3+p1*wc;

Kp(:,i) = (B1(:,i)*C2(:,i)-B2(:,i)*C1(:,i))/(A1(:,i)*B2(:,i)-A2(:,i)*B1(:,i));

Ki(:,i) = (A2(:,i)*C1(:,i)-A1(:,i)*C2(:,i))/(A1(:,i)*B2(:,i)-A2(:,i)*B1(:,i));

end

end

plot(Kp, Ki)

hold on

x = 0;

y = -0.03:0.0001:0.059;

plot(y,x,'k')

grid

axis([-0.04 0.07 -0.02 0.12])

xlabel ('Proportional Controller Gain K_{P}')

ylabel ('Integral Controller Gain K_{I}')

title ('Stability Region for \tau=0.5')

The result of the above-mentioned MATLAB code is:

4.2.2 STABILITY REGION CURVE FOR TIME DELAY=0.5s FOR VARIOUS PARTICIPATION FACTORS:

Fig 4.4 The plot of integral controller gain KI vs proportional controller gain Kp.

- For system to be stable:  The values of Kp and KI should lie within the curve boundary for that participation factors.

Example: For α1=0.3 and α0=0.7 the values are Kp =1 and KI =0.5.

Fig 4.5The plot of incremental frequency vs time for Kp =1 and KI =0.5.

- For system to be unstable:  The values of Kp and KI should lie anywhere out of curve boundary for that particular participation factors.

Example: For α1=0.3 and α0=0.7, the values are Kp =3 and KI =2.

Fig 4.6The plot of incremental frequency vs time for Kp =3 and KI =2.

- For system to be marginally stable: The values of Kp and KI should lie on or near to the curve boundary for that particular participation factors.

Example: For α1=0.3 and α0=0.7, the values are Kp =1.8 and KI =1.4.

Fig 4.7 The plot of incremental frequency vs time for Kp =1.8 and KI =1.4.

4.3.0 CONCLUSION:

Stability delay margins are computed for a wide range of PI controller gains. The stability delay margins are shown in Table 4.1 for EV participation factor of α1 = 0.2. The results in Table 4.1 indicate that for a fixed value of KP, the stability delay margin decreases when KI is increased, which infers a less stable system. The stability delay margin increases with KPfor nearly all values of KI. The theoretical delay margins are verified using timedomain simulations. The frequency response exhibits sustained oscillations which indicate the marginal stability of LFC-EV system. If the time delay exceeds the stability delay margin, the system will become unstable due to the growing oscillations in the frequency response.

The stability region in the (KP,KI )-plane is obtained for without EV aggregator and with three different EV aggregator participation factorsα1 = 0.1, 0.2 and 0.3, whereas the time delay is fixed at τ = 0.5 s. These participation factors imply that 10%, 20% and 30% of the required control efforts are provided by the EV aggregator with a time delay of τ = 0.5 s. Figure 4.4 compares the corresponding stability regions. Note that the largest region is observed when EV participation is not considered (α1 = 0). More importantly, the size of stability regions decreases as the EV participation factor increases, whereas the shape of the regions is unchanged. Figure 4.4 clearly illustrates that the stability regions get smaller as the contribution of EV aggregator to the frequency regulation increases due to the presence of communication time delay. The accuracy of stability boundary locus is validated by the time-domain simulations. It can be seen that the LFC-EV system is marginally stable due to undamped frequency response because of a complex conjugate root pair located on the jω-axis for the selected gains. This implies that the LFC-EV system will be asymptotically stable with decaying oscillation in the frequency response for any PI controller gains inside the region. The LFC-EV system will become unstable with growing oscillations in frequency for any controller gains outside the region.

Chapter 5:  Conclusion

This project work has presented a comprehensive study on the effect of integrating EV aggregator with communication time delay to conventional LFC system. For a given time delay and load sharing scheme, a set of all stabilizing PI controller gains that constitute a stability region in the parameter space of  the  controller  have  been  determined  using  a  graphical  ex‐act  method. The impact of both time the delay  and  EV aggregator participation factor on the stability regions has been evaluated. It is observed that the size of stability regions decreases as the time delay and EV participation factor increase.

To complement stability region results, stability delay margins  have  been  determined  for  a  large  number  of  PI  controller gains using a frequency domain  exact  method.  It  has  been observed that stability delay margin becomes smaller with  an  increase  in  the  integral  gain.  Moreover,  for  any  given  PI  controller  gains,  an  increase in  EV  aggregator  participation factor results in a decrease in stability delay margin.     If the PI controller gains and participation factor of EVs are not properly selected, the participation  of  EV  aggregator  with a communication time delay may cause instability and degrade the dynamic response against an  expectation  that  EVs can improve the LFC dynamic performance.

It is expected that the results will allow us to determine     the communication delay requirements and the design of PI controller for EV aggregators participating in frequency regulation service. In the future, the gain and phase margins will be taken into account for obtaining stability regions of load frequency control systems with multiple EVs aggregators and time delays to calculate all possibly purely complex roots and imaginary roots of characteristics equation. Also, the stability regions will be computed for multi-area load frequency control system. Along with that, computation of relative stability specifications will be included.

Chapter 6: References and Base Paper

1)      Ausnain Naveed Sahin Sonmez and Saffet Ayasun,’Impact of Electric Vehicle Aggregator with communication Time delay on Stability Regions and Stability Delay Margins in Load Frequency control System’, Journal of Modern Power Systems and clean Energy, In press, DOI: 10.35833/MPCE.2019,Springer. (BASE  PAPER).

2)    Hakan Gunduz, Sahin Sonmez and Saffet Ayasun, ‘Impact of Electric Vehicles Aggregator on the Stability Region Micro-Grid System with Communication Time Delay,’ 2019 IEEE Milan Power Tech.

3)    Vijay P. Singh, Nand Kishor, and Paulson Samuel, ‘Communication Time Delay Estimation for Load Frequency     Control in Two Area Power Systems,’ Ad Hoc Networks, Vol. 41, No. 1, pp. 69-85, May 2016.

4)    Ausnain Naveed, Sahin Sonmez and Saffet Ayasun, ‘Stability Regions in the Parameter Space for LFC System with EV Aggregator and Incommensurate Time Delays,’ 1st IEEE Global Power Energy and Communication Conference (GPECOM 2019), June 12-15, 2019, Cappadocia, Turkey.

5)   Han, Y., Zhang, K.,  Hong, L., Coelho, E. A. A., and Guerrero, J. M,  ‘MAS-based Distributed Coordinated Control and Optimization in Microgrid and Microgrid Clusters: A Comprehensive Overview.’ IEEE Transactions on Power Electronics, Vol. 33, No. 8, pp. 6488-6508, 2018.

6)   H. Luo, I. A. Hiskens and Z. Hu, ‘Stability Analysis of Load Frequency Control Systems With Sampling and Transmission Delay,’ IEEE Transactions on Power Systems, Vol. 35, No. 5, pp. 3603-3615, Sept. 2020

7)    K. S. Ko and D. K. Sung, ‘The Effect of EV Aggregators With Time-Varying Delays on the Stability of a Load Frequency Control System,’ IEEE Transactions on Power Systems, Vol. 33, No. 1, pp. 669-680, Jan. 2018

8)    Deniz Katipoglu, Sahin Sonmez and Saffet Ayasun, ‘Stability Delay Margin Computation of Load Frequency Control System with Demand Response,’ 1st IEEE Global Power Energy and Communication Conference (GPECOM 2019), June 12-15, 2019, Cappadocia, Turkey.

9)        H. Bevrani, A. Ghosh, and G. Ledwich, “Renewable energy sources and  frequency  regulation:  survey  and  new  perspectives,”  IET  Renew‐ able Power Generation, vol. 4, no. 5, pp. 438-457, Sept. 2010.

10)      W. Kempton and S. Letendre, “Electric vehicles  as  a  new  power  source for electric utilities,”  Transportation Research Part D, vol. 2,   no. 3, pp.157-175, Sept. 1997.

11)     J. R. Pillai and B. Bak-Jensen, “Integration of vehicle-to-grid in the western  Danish  power  system,”  IEEE  Transactions  on  Sustainable  Energy, vol. 2, no. 1, pp. 12-19, Jan. 2011.

12)      M. Y. Mu, J. Wu, J. Ekanayake et al., “Primary frequency response from electric vehicles in the Great Britain power system,”  IEEE Trans‐ actions on Smart Grid, vol. 4, no. 2, pp. 1142-1150, Jun. 2013.

13)       C. Guille and G. Gross, “A conceptual framework for the vehicle-to-  grid (V2G) implementation,” Energy Policy, vol. 37, no. 11, pp. 4379- 4390, Nov. 2009.

14)       S. Han, S. Han, and K. Sezaki, “Development of an optimal vehicle- to-grid aggregator for frequency regulation,” IEEE Transactions on Smart Grid, vol. 1, no. 1, pp. 65-72, Jun. 2010.

15)      R. J. Bessa and M. A. Matos, “The role of an aggregator agent for EV in the electrical market,”  in Proceedings of 7th Mediterranean Confer‐ ence and Exhibition on Power Generation, Transmission, Distribution and Energy Conversion, Agia Napa, Cyprus, Nov. 2010, pp. 7-10.

16)    R. J. Bessa, M. A. Matos, and F. J. Soares, “Framework for the partici‐ pation of EV aggregators in the electricity market,” in Proceedings of 2014  IEEE  International  Electric  Vehicle  Conference  (IEVC),  Flor‐ ence, Italy, Dec. 2014, pp. 1-8.

17)      A.  M.  Carreiroa,  H.  M.  Jorge, and  C.  H.  Antunesa,  “Energy  manage‐ ment systems aggregators: a literature survey,”  Renewable and Sustain‐ able Energy Reviews, vol. 73, pp. 1160-1172, Jun. 2017.

18)      C. Quinn, D. Zimmerle, and T. H. Bradley,  “The effect of communica‐tion architecture on the availability, reliability, and economics of plug-  in hybrid electric vehicle-to-grid ancillary services,” Journal of Power Sources, vol. 195, no. 5, pp. 1500-1509, Mar. 2010.

19)     K. Ko and D. K. Sung,  “The Effect of cellular network-based communication delays in an EV aggregator’ s domain on frequency regulation service,” IEEE Transactions on Smart Grid, vol. 10, no. 1, pp. 65-73, Jan. 2019.

20)     K. Mak and B. L. Holland,  “Migrating electrical power network SCA‐ DA systems to TCP/IP and ethernet networking,” Power Engineering Journal, vol. 16, no. 6, pp. 305-311, Dec. 2002.

21)     K. S. Ko and D. K. Sung, “The effect of EV aggregators with time- varying delays on the stability of a load frequency control system,”computation  of  quasi-polynomial  zeros,”  IEEE  Transactions  on  Auto‐ matic Control, vol. 54, no. 1, pp. 171-177, Jan. 2009.

22)     K. S. Ko, S. Han, and D. K. Sung, “A new mileage payment for EV aggregators with varying delays in  frequency  regulation  service,”  IEEE Transactions on Smart Grid, vol. 9, no. 4, pp. 2616-2624, Jul. 2018.

23)     H. Fan, L. Jiang, C. K. Zhang et al., “Frequency regulation of multi- area  power  systems  with  plug-in  electric  vehicles  considering  communication delays,” IET Generation, Transmission  &  Distribution,  vol. 10, no. 14, pp. 3481-3491, Nov. 2016.

24)   R. Sipahi and I. I. Delice, “Advanced clustering with frequency sweeping  methodology  for  the  stability  analysis  of  multiple  time-delay  sys‐ tems,” IEEE Transactions on Automatic Control, vol. 56, no. 2, pp. 467-472, Feb. 2011.

25)     I.  Delice  and  R.  Sipahi,  “Delay-independent  stability  test  for  sys‐ tems with multiple time-delays,” IEEE Transactions on Automatic Control, vol. 57, no. 4, pp. 963-972, Apr. 2012.